Deliverable 1: Basic Project Design: hardware design, back-end and front-end architecture.

Work Package 1.1: Hardware Schematic and Hardware specification: micro-controller, fingerprint sensor, real time clock (RTC), etc.

- Micro-controller: ESP32 DevKit V1

Characteristics:

- Processor: Dual-core Tensilica LX6 microprocessor, up to 240 MHz

- Memory:

  - RAM: 520 KB SRAM

  - Flash: Varies (commonly 4MB)

- Connectivity:

  - Wi-Fi: 802.11 b/g/n (STA, AP and dual mode)

  - Bluetooth: Bluetooth v4.2 BR/EDR and BLE

- GPIO Pins:

  - Digital I/O: 34 GPIO pins (various multifunction)

  - Analog Inputs: 18 (ADC1: GPIO32-39, ADC2: GPIO0, 2, 4, 12-15, 25-27, 34-39)

- Communication Interfaces:

  - UART: 3 (UART0, UART1, UART2)

  - SPI: 4

  - I2C: 2 (HSI and HSI2)

  - PWM: Multiple channels

- Power Supply:

  - Input Voltage: 5V via USB or VIN pin

  - Operating Voltage: 3.3V (logic level)

Functionality in Project:

- Acts as the central controller managing data acquisition from the fingerprint sensor, LCD display, RTC module, and communication with the remote server via Wi-Fi.

- Handles user authentication and data storage operations.

- Fingerprint Sensor: R305

Key Specifications:

- Interface: UART (Serial Communication)

- Operating Voltage: 3.3V to 6V (compatible with ESP32’s 3.3V logic)

- Resolution: 500 DPI (dots per inch)

- Precision :

  - FAR (False Acceptance Rate): < 0.001%.

  - FRR (False Rejection Rate): < 0.1%.

- Fingerprint Capacity: Up to 1000 fingerprints

- Capture Time: ~0.3 seconds

- Storage: Onboard memory for fingerprint templates

- LED Indicators: Status LEDs for operations (enrollment, verification)

Functionality in Project:

- Captures and reads users’ fingerprint data for authentication.

- Sends fingerprint data to the ESP32 for processing and verification against stored templates.

- Display LCD: 16x2 I2C

Characteristics:

- Type: Character LCD, 16 columns x 2 rows

- Interface: I2C via PCF8574 I/O expander

- Operating Voltage: 5V (logic levels are typically 5V compatible, but can work with 3.3V with proper configuration)

- Backlight: LED backlight (typically controlled via I2C)

- Contrast Control: Potentiometer or via I2C commands

- PINS:

  - SDA Pin: Data line

  - SCL Pin: Clock line

- Address: Common default is 0x27 or 0x3F

Functionality in Project:

- Displays system status, prompts, and other information.

- Provides real-time feedback to users during the authentication process.

- RTC Module: DS3231

Although the ESP32 microcontroller already has an integrated RTC, it may experience synchronization problems due to the lack of an internal battery as a power source. This is the main reason why we decided to use the DS3231 module. It is a low-cost, highly accurate I2C real-time clock with an integrated temperature-compensated crystal oscillator (TCXO) and crystal. The device incorporates a battery input and maintains accurate timekeeping even when the main power to the device is interrupted.

Characteristics:

- Interface: I2C

- Operating Voltage: 3.3V to 5V

- Accuracy: ±2 minutes per year

- Battery backup (for timekeeping when power is off)

- Timekeeping: Supports seconds, minutes, hours, day, date, month, and year.

- PINS:

  - SDA Pin: Data line

  - SCL Pin: Clock line

- Address: Typically 0x68

Functionality in Project:

- Keeps accurate real-time data for logging entry and exit times.

- Ensures time stamps are maintained even during power outages via the backup battery.

Power Supply: USB Current Adaptor

Characteristics:

- Output Voltage: 5V DC

- Current Rating: Minimum 2A

- Connector: USB Type-A or Micro/USB

Functionality in Project:

- Provides stable power to the ESP32 and connected peripherals.

- Ensures reliable operation of the system by maintaining adequate power supply.

Hardware Schematics

The system will have the following connections as shown in the figure:

- ESP32:

  - Power from USB.

  - I2C bus for LCD and RTC:

    - SDA (GPIO21), SCL (GPIO22).

  - UART for fingerprint sensor:

    - TX (GPIO1), RX (GPIO3).

- Fingerprint Sensor (R305):

  - VCC → ESP32 3.3V or 5V.

  - GND → ESP32 GND.

  - TX → ESP32 RX (GPIO16).

  - RX → ESP32 TX (GPIO17).

- LCD 16x2:

  - VCC → ESP32 5V.

  - GND → ESP32 GND.

  - SDA → ESP32 GPIO21.

  - SCL → ESP32 GPIO22.

- RTC DS3231:

  - VCC → ESP32 3.3V or 5V.

  - GND → ESP32 GND.

  - SDA → ESP32 GPIO21.

  - SCL → ESP32 GPIO22.